Braille Note Tutorial By Katie Beaver Module Two Creating a Document Documents

are created using the Word Processing program called "Keyword" Turn on the

Braille Note If you are not at the Main Menu, go there by pressing 1,2,3,4,5,6

space Choosing the Word Processor Using the space or the advance key, move to

the Word Processor. Select the Word Processor by pressing the Enter key or the

Next/Select Key. Navigating the Keyword Menu: Using the space or the advance

key, read all the keyword menu items. Move to "create a document" and select it

by pressing the Enter key or the Next/Select Key. Setting up a new docuement:

The Braille Note says "Folder name? Press enter for GENERAL" Press ENTER to

choose this folder. Note: If the Braille Note did not say "General" but instead

said a different name, press space to cycle through the available folders until

you hear General, and then press enter choose it. Naming a document: The Braille

Note says "Document to create?" - Type your first name in grade 2 and press

Enter The Braille Note says "Top of Document. Blank." You are ready to begin

typing. The Cursor: As you begin to type, the cursor, which represents your

typing position, will be displayed as dots 7 and 8. Typing sentences: Type the

following sentence keeping in mind that if you make a typing mistake, simply

press the backspace key Now is the time for all good men to come to the aid of

their country. Next, type the sentence, This is fun. Moving the cursor: The

cursor is moved and speaking commands are executed by using specific braille

input keys together with the spacebar. The cursor can also be moved by using the

"Touch Cursor Buttons" located directly behind each braille cell Simply locate

the cell where you want the cursor and press the touch cursor on that cell When

moving the cursor and listening to a file, remember that the keys on the left

side move back and keys on the right side move forward Top of Document: Dots

1,2,3 and space moves the cursor to the top of the document The Braille Note

says "top of document" and speaks the first word Moving by characters 3 and 6

Dot 6 with space moves forward and speaks one character at a time Dot 3 with

space moves back and speaks one character at a time Dots 3,6 and space speaks

the current character Phonetic pronunciation of a character (3 and 6) Pressing

dots 3,6 and space twice in a row will expand the letter into a word, such as A

- Alpha, B - Bravo Moving by words 2 and 5 Dot 5 with space moves forward and

speaks one word at a time Dot 2 with space moves back and speaks one word at a

time Dots 2,5 and space speaks the current word Spelling the current word:

Pressing Dots 2,5 and space twice in a row will spell the current word To spell

the word letter by letter with the contractions expanded, press dots 2,5, space

a third time Moving by sentences 1 and 4 Dot 4 with space moves forward and

speaks one sentence at a time Dot 1 with space moves back and speaks one

sentence at a time Dots 1,4 and space speaks the current sentence Bottom of

Document: Dots 4,5,6 and space moves the cursor to the bottom of the document

The Braille Note says "bottom of document" and then says "blank" Listening to

the whole file: Move to the top of document by pressing dots 1,2,3 space Press

space with the letter G for "go" (dots 1,2,4,5) Backspace and Enter together

will stop the speech Review To move one character at a time use space with dot 3

or dot 6 To move one word at a time use space with dot 2 or dot 5 To move one

sentence at a time use space with dot 1 or dot 4. Turning the Speech On and Off

Press Space and Previous together to cycle through the three speech output

options: 1. Speech on request 2. Speech off 3. Speech on Reading the file in

braille: The Braille Note can display 18 or 32 characters at a time, depending

on which model you are using To facilitate reading, the Braille Note avoids

breaking the last word by leaving the last few cells blank Press space with dots

1,2,3 to move to the top of the document The thumb keys are used to refresh the

display Advancing the Display: Read the braille display Press the Advance key to

advance the braille to the next section of text Continue to press Advance and

read the entire document Going Backwards: Press the Back key to move the braille

back one section of text at a time Touch Cursor Remember that you can move the

cursor any place within the display by using the touch cursor buttons. Turning

the Braille Display On & Off Press Space and Next together to turn the braille

display on and off Moving the Display: You can also move the display to the left

or right a word at a time Press Previous with Advance to move the display a word

to the right Press Previous with Back to move the display a word to the left

Practice: Go to the top of the document Listen to the document from the

beginning to the end, word by word Move the cursor to the word “country” Spell

the word Practice listening & reading the document When you are done, turn the

Braille Note off by first going to the Main Menu.