Braille Note Tutorial By Katie Beaver Module Seven About the Disk Drive The

SuperDisk drive plugs into the PC card slot on the right side of the Braille

Note. It supports standard 3.5 and larger capacity super disks. Feel along the

right side of the Braille Note and you will find a horizontal slot. The card

eject button is located in a recess behind the slot. Why use the Disk Drive You

can use the disk drive to: Back up valuable data Store very big documents

Exchange files with computers The SuperDisk Drive The disk drive must be plugged

in to work. You can connect the disk drive with the Braille Note on or off. If

the Braille Note is on, it will say "card inserted." Do not turn off the Braille

Note or eject the PC card if the disk drive motor is running. Connecting the

Disk Drive If the Braille Note is off, turn it on. Plug in the disk drive power

adapter. Insert the PC Card into the card slot on the right side of the Braille

Note. It can only go in one way. If it doesn't insert all the way, turn the card

over and try again. Braille Note will say, "card inserted" Copying files from

the Braille Note to a Disk Copying a file does not remove or change the original

file. It simply creates a backup. The file that you copy in this process is a

"KeySoft" braille file that can only be read by a Braille Note. Importing and

exporting files allows you to change the format to enable file exchange with

computers. Insert a disk into the disk drive From Main Menu, choose File Manager

Menu (F) From File Manager, choose Copy File (C) When asked "Source drive, press

enter for ..." If it says "flash disk" press enter. Otherwise, press space to

get the drive list and choose "flash disk" from the list. Navigate through the

folders and files to select the file you want to copy, then press enter. When

asked "Destination drive, press enter for..." If it says "storage card" press

enter. Otherwise, press space to get the drive list and choose "storage card"

from the list. Copying Files from Email to a Floppy Disk Open the email that

contains the attached files you want to copy. Insert an empty, preformatted disk

into your computer. Right click on an attached file. Left click on "save as"

Change the SAVE IN selection at the top of the dialog box to 3 1/2 floppy (A:),

then press enter. Repeat these steps for each file until you have all the files

you want copied on the floppy. Insert the floppy into the Braille Note disk

drive. Wait until the drive stops spinning. Importing files from a disk to the

Braille Note From Main Menu, choose File Manager Menu (F) From File Manager,

choose Translation Menu (T) From Translation Menu choose Import (I) When asked

"import from which device?" press F for File When prompted for drive, press

space to get the drive list. Press space until the Braille Note says, "storage

card." Press enter When Braille Note says, "directory name? Press enter for

slash. Computer braille is required," press enter When Braille Note says, "file

to import? Computer Braille is required," type the following: Dots 1,6 (This is

a star) Dots 4,6 (This is a period) Dots 1,6 (This is a star) Press enter. When

Braille Note says, "translate into braille?" type the letter Y When Braille Note

says, "destination folder name? Press enter for ..." Press enter if the folder

name is correct. If not, press space until you hear the correct folder, then

press enter. If there is only one file on the disk, the Braille Note will say,

"Braille document name?" You will need to type in a name However, if you have

multiple files on the floppy, you will not be prompted for a file name. When

finished, you return to the translation menu. Go back to Main Menu. Exporting

files from the Braille Note to a disk From Main Menu, choose File Manager Menu

(F) From File Manager, choose Translation Menu (T) From Translation Menu choose

Export (E) When asked "export document from which folder? Press enter for ..."

press enter if the file you are exporting is in the folder identified. If not,

press space to get to the folder you want. When Braille Note says, "document

name? Press enter for ..." navigate to the file name you want and press enter

When Braille Note says, "export to which device?" press F When asked, "export as

which file type? ASCI Text File" Press st (dots 3,4) with space repeatedly to

cycle through the available file formats. Press enter on the format you want.

(plain braille, Microsoft word, rich text format, etc.) When Braille Note says,

"destination drive? Press enter for ..." Navigate to "storage card" if

necessary, then press enter. When Braille Note says, "directory name? Press

enter for slash. Computer braille is required," press enter When Braille Note

says, "destination file name? Computer braille is required" type in a file name

and press enter. Braille Note says, "One file exported. Translation Menu." You

can now remove the disk, put it in the computer and open it in whatever program

matches the format (for example Microsoft Word)